802.11g Wireless

PCI Card

Quick Installation Guide

REGULATORY STATEMENTS

FCC Certification

The United States Federal Communication Commission (FCC) and the Canadian Department of Communications have established certain rules governing the use of electronic equipment.

Part15, Class B

This device complies with Part 15 of FCC rules. Operation is subject to the following two conditions:

1) This device may not cause harmful interface, and

2) This device must accept any interface received, including interface that may cause undesired operation. This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to Part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference in a residential installation. This equipment generates, uses and can radiate radio frequency energy, and if not installed and used in accordance with the instructions, may cause harmful interference to radio communications. However, there is no guarantee that interference will not occur in a particular installation. If this equipment does cause harmful interference to radio or television reception, which can be determined by turning off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

- Reorient or relocate the receiving antenna.

- Increase the distance between the equipment and receiver.

- Connect the equipment into an outlet on a circuit different 
 from that to which the receiver is connected.

CAUTION:

- To comply with FCC RF exposure compliance requirements, a separation distance of at least 20 cm must be maintained between the antenna of this device and all persons.

- This transmitter must not be co-located or operating in conjunction with any other antenna or transmitter.

# Network Configuration

The 802.11g Wireless PCI Card is an IEEE802.11g-compliant PCI Type DSSS wireless LAN PCI adapter. It fully supports wireless networking under Windows 98SE/ME/2000/XP.

The 802.11g Wireless PCI Card can be operated in Ad-Hoc or Infrastructure network configurations. Ad-Hoc mode allows 802.11g Wireless PCI Card users to join a Basic Service Set (i.e., peer-to-peer mode, without access point). Infrastructure mode allows 802.11g Wireless PCI Card users to join an Extended Basic Service Set (i.e., connect to an Access Point)

# Installing the Device

### In Windows 98SE/ME

- Insert the 802.11g Wireless LAN PCI Card into the PCI slot on your notebook and start Windows. Windows will auto-detect new hardware and will display an “Add New Hardware Wizard” window.

- Select “Search for the best driver for your device (recommended)”. Insert the CD-ROM that came with this device into the CD-ROM drive. Specify the location where the driver is placed. Click on Next to install the driver.

- The windows will find the device and display the description of the device. Click on Next to continue.

- Once the [Please insert the disk labeled “Windows 98SE/ME CD-ROM”, and then click OK] window appears, enter the path corresponding to the appropriate drives and click OK. Usually these files can be found at C:Windows or C:Windows\system.

- Click Finish to complete the installation. Restart Windows.

### In Windows 2000

- Insert the 802.11g Wireless PCI Card into the PCI slot on your notebook and start Windows. Windows will auto-detect the Wireless LAN PCI Adapter and a “Found New Hardware Wizard” window will show up.

- Select “Search for a suitable driver for my device (recommended)”. Insert the Product CD-ROM into the appropriate drive. Click on Next to install the driver.

- The windows will find the device and display the description of the device. Click on Next to continue.

- Click Finish to complete the installation

### In Windows XP

- Insert the 802.11g Wireless PCI Card into the PCI slot on your notebook and start Windows. Windows will auto-detect the Wireless LAN PCI Adapter and a “Found New Hardware Wizard” window will show up.

- Select “Install from a list of specific location (Advanced)” and insert the CD-ROM that came with this device into the appropriate drive. Click on Next to install the driver.

- Select “Search removable media (floppy, CD-ROM…) ”, click on Next to install the driver.

- The Windows XP compatibility screen will show up. Please click Continue Anyway button to continue.

- Click Finish to complete the installation.

After installing the Wireless LAN PC Card, the Windows XP will display a “Wireless Network Connection # ” message.

Click on the message and the “Automatic Wireless Network Configuration” will then appear automatically. You may click on Connect button to allow users to connect to an available wireless infrastructure network (Access Point), shown as follows:

You may click the Advanced button to perform advanced configuration for the 802.11g Wireless PCI Card, shown as below.

Warning: You must choose one way to configure the 802.11g Wireless PCI Card either of using our WLAN Utility by un-checking this check box or using Windows XP Automatic Wireless Network Configuration first by checking this check box.

For more information on using the Automatic Wireless Network Configuration please refer to Windows XP Help file.

However, the WLAN Utility, which came with the 802.11g Wireless PCI Card, provides you more tools to configure the 802.11g Wireless PCI Card and to monitor the wireless connection. For more information on installing and using the WLAN Utility, please refer to the sections titled “Installation of the Wireless LAN Utility” and “Usage of the WLAN Utility” in the user manual.

Note: To use the WLAN Utility under Windows XP, you need to disable the Automatic Wireless Network Configuration first. Steps are described as follows:

# Installing the Utility

- Insert the device CD-ROM into the appropriate drive. Go to the utility folder and click setup.exe. The opening screen will show up. Click Continue to continue.

- Follow the on-screen instructions to install the Wireless LAN Utility. Click Next.

- Upon completion, go to Program Files and run the Wireless LAN Utility. The utility interface will then appear and at the same time its icon appears in the System Tray in the bottom right corner of your task bar.

## Usage of the WLAN Utility

The WLAN Utility consists of window with 3 items for you to monitor and configure the 802.11g Wireless PCI Card: Configuration, Site Survey and About.

Configuration:

The Configuration item allows you to modify the configuration parameters for the 802.11g Wireless PCI Card such as Profile, SSID, Network Type, AdHoc Channel, Transmit Rate, WEP, RTS, Fragment Threshold and Power Save mode. Furthermore, you may monitor the current status of the 802.11g Wireless PCI Card as State, Current Channel, Current Tx Rate, Throughput, Link Quality and Signal Strength.

Site Survey:

By clicking the Rescan button, the Site Survey can display Access Points around the working environment. Besides showing the ESSID of each Access Point, it also displays BSSID, Channel, Network Type, Encryption, Signal, and Rates. To join any of the displayed Access Points, highlight the Access Point you desire to connect and then click the Join.

About:

The About item shows the versions of the WLAN Utility, driver, and firmware of this device. Also, the MAC address and frequency domain are displayed.

## Quick Tips to Configure the Wireless LAN PCMCIA Card

This Quick Start Guide covers the basic features needed to use the 802.11g Wireless PCI Card. Follow these steps to configure network settings.

### Set the Profile (Optional)

The Profile field allows you to set values for all parameters by selecting a previously defined profile. To create the profile, go to the Profile field, type a profile name and set the corresponding parameters. After changing parameters, save the profile and click the Apply button to take effect.  You can have multiple profiles and modify the profile at any time.

You may also see the profile from the WLAN Utility icon that appears in the System Tray in the bottom right corner of your task bar by right clicking the WLAN Utility icon or you may choose  the profile by directly right clicking the system tray icon on the task bar.

### Set the Network Type

From the Main Menu, click the Configuration tab, go to Network Type and click the down arrow at the right of the Network Type field. You will then see 3 network types for the 802.11g Wireless PCI Card to operate. If you need to access company network or the Internet via Access Point, select “Infrastructure”. If standard “802.11 Ad Hoc” is selected, you need to set wireless stations with the same SSID.

### Set the SSID

The SSID is the unique ID used by Access Points and stations to identify a wireless LAN. Wireless clients associating to any Access Point must have the same SSID. The default setting is ANY, which allows the 802.11g Wireless PCI Card to automatically associate to any Access Point (Infrastructure mode) in the vicinity of your wireless adapter. The SSID can be set up to 32 characters and is case sensitive.

### Set the Channel (for Ad-Hoc network)

To form AdHoc network, the same channel is required. Before choosing a channel, it is advised to use the Site Survey tool to check the quality of each channel. Afterwards, go to the Channel field and choose a clear channel for your wireless LAN stations. Click the Apply button to let the change take effect.

### Set the Encryption:

To prevent unauthorized wireless stations from accessing data transmitted over the network, the 802.11g Wireless PCI Card offers highly secure data encryption, known as WEP (Wired Equivalent Privacy). The Encryption tab allows you enable encryption and set the encryption keys, making your data transmission over the air more secure. To activate the WEP Encryption, go to the Configuration tab and check the WEP Enabled check box. An Encryption window will then appear.

From the WEP encryption item, pull down the menu and you will see two methods to set the WEP keys which are described below:

- Create Encryption Keys by Using a Passphrase

To create encryption keys by using a passphrase, click the Create Key with Passphrase check box and type a character string in the Passphrase field. As you type, the utility uses an algorithm to generate 4 keys automatically. Select either the 64bit or 128bit encryption first and type a string in the Create Key with Passphrase field. Select one key from the 4 WEP keys and click OK. Then click the Apply button on the Configuration tab to let the setting take effect.

- Warning: When Create Key with Passphrase is enabled, the wireless device’s Key with PassPhrase must match the Key with PassPharse used by the access point with which wireless device are planning to communicate.

- Create Encryption Keys Using a Passphrase (128-bit)

- Create Encryption Keys Manually

You can also create up to 4 encryption keys manually by clicking the Create Keys Manually check box.

For 64bit encryption you may choose:

- Alphanumeric: 5 characters (case sensitive) ranging from “a-z”, “A-Z” and “0-9” (e.g. MyKey)

- Hexadecimal: 10 hexadecimal digits in the range of “A-F”, “a-f” and “0-9” (e.g. 11AA22BB33)

For 128bit encryption you may choose:

- Alphanumeric: 13 characters (case sensitive) ranging from “a-z”, “A-Z” and “0-9” (e.g. MyKey12345678)

- Hexadecimal: 26 hexadecimal digits in the range of “A-F”, “a-f” and “0-9” (e.g. 00112233445566778899AABBCC).

After entering the WEP keys in the key field, select one key as active key, click the OK button and then click the Apply button to let the setting take effect.